TK003 GPS Tracker SMS Commands:

| Functions | SMS Command Format | Command Reply |
| --- | --- | --- |
| Restart | RESET# | RESET OK. |
| APN setup | APN,cmnet,,# APN,cmnet,aaa,bbb# | APN:cmnet,aaa,bbb |
| Map Link | URL# | http://maps.google.com/maps?q=+22.601862,+113.832383 |
| Status Check | STATUS# | Battery:4.23V;GPRS:Offline;GSMSignal Level:25;ACC:OFF;GPS:OFF;Defense:ON; |
| Fuel control | RELAY,1# (Cut off fuel) RELAY,0# (Resume fuel) | RELAY 1 OK |
| GPRS interval set | TIMER,T1,T2# T1=10~60 secs, ACC ON. T2=30~1800 secs, ACC OFF | Send: TIMER,30,300# Reply: TIMER ACC ON:30s,ACC OFF:300s |
| Shock alarm ON | SENALM,ON,M# M=0, GPRS alarm M=1, SMS+GPRS alarm M=2, GPRS+SMS+Call alarm | Send: SENALM,ON,2# Reply: SENALM:ON,2 |
| Shock alarm OFF | SENALM,OFF# | SENALM:OFF |
| Tamper alarm ON | POWERALM,ON,M,T1,T2# M=0, GPRS alarm M=1, SMS+GPRS alarm M=2, GPRS+SMS+Call alarm T1=2~60 second T2=1~3600 second | Send: POWERALM,ON,2,10,10,# Reply: POWERALM:ON,2,10,10 T1: tamper lasting time T2: minimum charging time |
| Tamper alarm OFF | POWERALM,OFF# | POWERALM:OFF |
| Speed alarm ON | SPEED,ON,B,C,M# B=5~600 secs C=1~255 km/h M=0, GPRS alarm M=1, SMS+GPRS alarm M=2,GPRS+SMS+Call | Send:SPEED,ON,10,120,1# Reply:SPEED:ON,10,120,1 |
| Speed alarm OFF | SPEED,OFF# | SPEED:OFF |
| ACC ON alarm | ACCALM,ON,M# M=1, SMS M=2, Call M=3, SMS+Call | Send:ACCALM,ON,2# Reply:ACCALM:2,OK |
| ACC ON alarm OFF | ACCALM,OFF# | ACCALM,OFF OK |
| Set Admin number | CENTER,A,TEL# | CENTER,A,15012440563# |
| Delete number | CENTER,D# | CENTER Del OK |